INDUSTRIAL ENVIRONMENT
MONITORING SYSTEM

ESP32 + AWS IoT Core + Lambda + DynamoDB + React Dashboard

Complete Project Report
End-to-end documentation, architecture, deployment & operations

Prepared by: Deepak
Document version: 1.0
Project status: Production-ready · End-to-end latency ~2-3 s

ESP32  ──MQTT/TLS──►  AWS IoT Core
   │                          │
   └──HTTPS POST──►  API Gateway  ──►  Lambda  ──►  DynamoDB
                                                       │
                                                       ▼
                                               React / HTML Dashboard

# Contents

1. Executive Summary	Page 3

2. Project Overview	Page 3

3. System Architecture	Page 4

4. Hardware Bill of Materials	Page 5

5. Software Stack	Page 6

6. ESP32 Firmware	Page 7

7. AWS Cloud Setup	Page 9

8. Dashboard	Page 12

9. Build & Deployment	Page 13

10. Performance & Latency	Page 14

11. Cost Analysis	Page 15

12. Security	Page 15

13. Testing & Verification	Page 16

14. Troubleshooting	Page 17

15. Future Enhancements	Page 18

16. Project Deliverables	Page 19

# 1. Executive Summary

This document describes the design, implementation and operation of an end-to-end Industrial Environment Monitoring System (IEMS). The system pairs an ESP32 microcontroller running ESP-IDF v6.x firmware with four physical sensors (temperature, humidity, air quality, flame), securely streams the readings to Amazon Web Services using two redundant cloud channels (MQTT/TLS and HTTPS), persists every reading in DynamoDB, and renders live data on a browser-based dashboard within 2-3 seconds of acquisition.

The system is deployed, tested and operating end-to-end. Every loop of the firmware produces a JSON payload that is acknowledged by the AWS IoT MQTT broker AND posted to the API Gateway endpoint AND persisted to DynamoDB AND rendered on the dashboard, with full observability through CloudWatch logs.

Key outcomes:

End-to-end latency from sensor read to dashboard render: ~2-3 seconds.

Mutual TLS authentication using X.509 certificates - no shared secrets on the wire.

Serverless cloud architecture - zero infrastructure to maintain or patch.

Estimated operating cost: ~$5/month per device at 2-second sample rate.

Auto-pruning storage via DynamoDB TTL (30-day retention by default).

Hardware bill of materials under ₹600 per device.

# 2. Project Overview

## 2.1 Problem Statement

Industrial facilities (factories, server rooms, chemical plants, storage warehouses) require continuous monitoring of environmental conditions to ensure safety, regulatory compliance and operational efficiency. Traditional SCADA solutions are expensive, proprietary and slow to deploy. There is a strong demand for a low-cost, modern, cloud-native alternative that any plant operator can stand up in a few days.

## 2.2 Goals

Real-time visibility of temperature, humidity, air quality and fire status.

End-to-end latency under 3 seconds.

Secure, authenticated data transport (mutual TLS).

Cloud-native, serverless backend that scales to thousands of devices without re-architecture.

Browser-accessible dashboard - no native app installs.

Persistent storage of every reading for historical analysis and compliance audit.

## 2.3 Non-Goals (Out of Scope for v1)

Battery operation (currently USB-powered).

OTA firmware updates (planned for v2).

Multi-region failover (planned for v2).

User authentication on the dashboard (planned for v2).

# 3. System Architecture

The system follows a classic device-to-cloud-to-browser pipeline, with two redundant cloud-ingress paths so that a failure in either API Gateway or AWS IoT Core does not interrupt data flow.

┌─────────┐
│  ESP32  │  reads sensors every 2 s
│         │  DHT11 (temp/humidity)
│         │  MQ135 (air quality, ADC)
│         │  Flame sensor (digital)
└────┬────┘
     │ builds JSON: {temperature, humidity, air_quality, fire}
     │
     ├──────► MQTT publish over TLS (port 8883)
     │           topic: environment/data
     │           ▼
     │       AWS IoT Core ─►(IoT Rule, optional)─► Lambda ─► DynamoDB
     │
     └──────► HTTPS POST (port 443)
                 ▼
             API Gateway (HTTP API v2)
                 │
                 ▼
             Lambda (environment_api)
                 │
                 ▼
             DynamoDB table EnvironmentReadings
                 ▲
                 │ GET /prod/data?limit=30 every 1 s
                 │
             React / HTML Dashboard (browser)

## 3.1 Data flow per cycle

ESP32 samples DHT11, MQ135 and the flame sensor (~ 15 ms).

ESP32 formats a compact JSON payload (~70 bytes).

Payload is published to AWS IoT Core over MQTT/TLS (QoS 1) - acknowledged in ~1 s.

In parallel, payload is POSTed to API Gateway over HTTPS - acknowledged in ~1-3 s.

API Gateway invokes the environment_api Lambda function.

Lambda writes one row to the DynamoDB table EnvironmentReadings.

Dashboard, polling every 1 s, calls GET /prod/data?limit=30.

Lambda queries DynamoDB and returns the latest reading + 29 historical rows.

Dashboard re-renders cards and table within the next animation frame.

# 4. Hardware Bill of Materials

| Component | Specification | Qty | Pin (ESP32) | Approx. Cost (INR) |
| --- | --- | --- | --- | --- |
| ESP32 Dev Board | ESP32-WROOM-32, 4 MB flash, 38-pin | 1 | — | ₹350 |
| DHT11 | Temperature + humidity digital sensor | 1 | GPIO 4 | ₹70 |
| MQ135 | Air-quality gas sensor (analog) | 1 | GPIO 34 (ADC1 ch6) | ₹90 |
| Flame Sensor | IR flame-detection digital (active LOW) | 1 | GPIO 5 | ₹50 |
| Jumper wires | Female-to-female, 20-cm pack | 1 | — | ₹30 |
| Breadboard | 830-point | 1 | — | ₹70 |
| Total |  |  |  | ≈ ₹660 |

## 4.1 Wiring diagram (text form)

ESP32 3V3  ─►  DHT11 VCC
ESP32 GND  ─►  DHT11 GND
ESP32 GPIO4 ─► DHT11 DATA          (+ 10 kΩ pull-up to 3V3, if module lacks it)

ESP32 3V3  ─►  MQ135 VCC
ESP32 GND  ─►  MQ135 GND
ESP32 GPIO34 ── MQ135 AO            (ADC1 channel 6, oneshot)

ESP32 3V3  ─►  Flame VCC
ESP32 GND  ─►  Flame GND
ESP32 GPIO5 ── Flame DO             (digital, active-LOW)

# 5. Software Stack

| Layer | Technology | Version | Role |
| --- | --- | --- | --- |
| Firmware framework | ESP-IDF (Espressif IoT Development Framework) | v6.0.1 | Native C SDK for ESP32. Production-grade, not Arduino. |
| Toolchain | xtensa-esp-elf-gcc | 15.2.0 | Compiles firmware for ESP32 (Xtensa LX6). |
| MQTT client | espressif/mqtt managed component | 1.0.0 | MQTT 3.1.1 client with TLS support. |
| DHT driver | esp-idf-lib/dht | 1.2.0 | Bit-banged driver for DHT11/22 sensors. |
| TLS | mbedTLS (bundled with ESP-IDF) | — | TLS 1.2 / 1.3 with hardware AES, SHA, MPI acceleration. |
| Cloud broker | AWS IoT Core | — | Managed MQTT broker, X.509 mutual auth, rule engine. |
| Compute | AWS Lambda | Node.js 24.x | Stateless function for write & read APIs. |
| Storage | Amazon DynamoDB | — | NoSQL key-value store with TTL auto-prune. |
| API | Amazon API Gateway (HTTP API v2) | — | REST endpoint, CORS, automatic deploy stages. |
| Dashboard | React 18 + Vite 5 (and zero-build HTML alt) | — | Single-page app polling the API every 1 s. |
| CLI / DevOps | AWS CLI v2 + PowerShell | — | Lambda upload, DynamoDB ops, log tailing. |

# 6. ESP32 Firmware

## 6.1 Project layout

environment_monitor/
├── CMakeLists.txt              # Top-level project file
├── sdkconfig.defaults          # Pinned ESP-IDF config
├── main/
│   ├── CMakeLists.txt          # main component; EMBED_TXTFILES certs
│   ├── idf_component.yml       # esp-idf-lib/dht, espressif/mqtt, espressif/cjson
│   ├── main.c                  # app_main, sensors, MQTT, HTTPS
│   ├── wifi.c, wifi.h          # Wi-Fi station bring-up
│   ├── AmazonRootCA1.pem       # AWS server CA  (public)
│   ├── device.pem.crt          # Device certificate (private)
│   └── private.pem.key         # Device private key (private)

## 6.2 Key configuration constants

| Constant | Value | Meaning |
| --- | --- | --- |
| WIFI_SSID / WIFI_PASS | (set in wifi.c) | Local Wi-Fi credentials |
| AWS_ENDPOINT_URI | mqtts://a2aazipzc1zlp2-ats.iot.us-east-1.amazonaws.com | AWS IoT MQTT endpoint |
| AWS_ENDPOINT_PORT | 8883 | TLS-MQTT port |
| MQTT_TOPIC | environment/data | Publish topic |
| MQTT_CLIENT_ID | esp32-env-monitor-01 | Per-device unique ID |
| API_URL | https://srb2rp6ww8.execute-api.us-east-1.amazonaws.com/prod/data | Dashboard write API |
| DHT_GPIO | GPIO 4 | DHT11 data line |
| FIRE_GPIO | GPIO 5 | Flame sensor DO |
| MQ135_CHANNEL | ADC1 channel 6 (GPIO 34) | Air-quality analog input |
| SAMPLE_PERIOD_MS | 2000 | Cycle interval (DHT11 minimum is 1 s) |

## 6.3 Per-cycle execution (pseudo-code)

void app_main(void) {
    wifi_connection();      // blocks until IP acquired
    mqtt_start();           // launches MQTT client task
    sensors_init();         // ADC oneshot + GPIO config
    while (1) {
        // 1) Sensor reads
        dht_read_float_data(DHT_TYPE, DHT_GPIO, &humidity, &temperature);
        adc_oneshot_read(s_adc_handle, MQ135_CHANNEL, &mq135_raw);
        int air_quality = (mq135_raw * 500) / 4095;
        int fire        = (gpio_get_level(FIRE_GPIO) == 0) ? 1 : 0;

        // 2) Build JSON
        snprintf(json_data, sizeof json_data,
                 "{\"temperature\":%.2f,\"humidity\":%.2f,\"air_quality\":%d,\"fire\":%d}",
                 temperature, humidity, air_quality, fire);

        // 3) Cloud paths
        esp_mqtt_client_publish(s_mqtt_client, MQTT_TOPIC, json_data, 0, 1, 0);
        send_to_dashboard(json_data);

        vTaskDelay(pdMS_TO_TICKS(SAMPLE_PERIOD_MS));
    }
}

## 6.4 Security - mTLS handshake

Every MQTT connection performs a mutual TLS handshake. The ESP32 presents its X.509 device certificate (device.pem.crt) signed by AWS IoT, and the broker presents the Amazon Root CA 1 certificate which we have embedded in the firmware binary. Both sides verify the other before any application payload is exchanged. There are no shared secrets, no passwords and no static API tokens on the device.

# 7. AWS Cloud Setup

## 7.1 AWS IoT Core

We create one Thing per device, attach one X.509 certificate to each Thing, and attach a least-privilege policy to each certificate.

Policy (least-privilege):

{
  "Version": "2012-10-17",
  "Statement": [
    { "Effect": "Allow", "Action": "iot:Connect",
      "Resource": "arn:aws:iot:us-east-1:*:client/esp32-env-monitor-*" },
    { "Effect": "Allow", "Action": "iot:Publish",
      "Resource": "arn:aws:iot:us-east-1:*:topic/environment/data" },
    { "Effect": "Allow", "Action": "iot:Subscribe",
      "Resource": "arn:aws:iot:us-east-1:*:topicfilter/environment/*" },
    { "Effect": "Allow", "Action": "iot:Receive",
      "Resource": "arn:aws:iot:us-east-1:*:topic/environment/*" }
  ]
}

## 7.2 DynamoDB

Table schema:

| Attribute | Type | Role | Example |
| --- | --- | --- | --- |
| device_id | String | Partition key | esp32-env-monitor-01 |
| timestamp | Number | Sort key (epoch ms) | 1748358912388 |
| temperature | Number | Sensor reading | 28.10 |
| humidity | Number | Sensor reading | 66.00 |
| air_quality | Number | Sensor reading (0-500) | 17 |
| fire | Number | 0 = clear, 1 = fire | 0 |
| ttl | Number | Auto-delete (epoch s) | 1750950912 |

TTL is configured on the ttl attribute (30-day retention). DynamoDB automatically deletes expired rows at no extra cost. Billing is PAY_PER_REQUEST so there is no capacity to plan.

## 7.3 Lambda - environment_api

Single Node.js 24.x function with the handler index.handler. It branches on HTTP method:

| Method | Source | Action |
| --- | --- | --- |
| POST | ESP32 (HTTPS) or AWS IoT Rule | Validate payload, insert one row into EnvironmentReadings. |
| GET | Dashboard (browser) | Query the latest N rows for the device and return JSON. |
| OPTIONS | Browser CORS preflight | Return CORS headers. |

Environment variables: TABLE_NAME (default EnvironmentReadings), DEFAULT_DEVICE_ID (default esp32-env-monitor-01), MAX_HISTORY (default 50), TTL_DAYS (default 30). Timeout: 10 seconds. Memory: 128 MB.

IAM permissions:

{
  "Version": "2012-10-17",
  "Statement": [{
    "Effect": "Allow",
    "Action": ["dynamodb:PutItem", "dynamodb:Query", "dynamodb:GetItem"],
    "Resource": "arn:aws:dynamodb:us-east-1:077463315120:table/EnvironmentReadings"
  }]
}

## 7.4 API Gateway (HTTP API v2)

| Setting | Value |
| --- | --- |
| API name | environment-api |
| API type | HTTP API (v2) |
| Stage | prod (auto-deploy enabled) |
| Route 1 | POST /data → environment_api Lambda |
| Route 2 | GET /data → environment_api Lambda |
| CORS Allow-Origin | * |
| CORS Allow-Methods | GET, POST, OPTIONS |
| CORS Allow-Headers | content-type |
| Invoke URL | https://srb2rp6ww8.execute-api.us-east-1.amazonaws.com/prod/data |

## 7.5 (Optional) IoT Rule - cloud-side write path

For full redundancy we can create an IoT Rule that forwards every MQTT message on environment/data straight into the same Lambda. This way the device's HTTPS POST becomes belt-and-braces - if API Gateway is unhealthy, MQTT alone still gets data into DynamoDB.

{
  "ruleName": "EnvironmentDataToLambda",
  "topicRulePayload": {
    "sql": "SELECT *, topic() as mqtt_topic, timestamp() as timestamp FROM 'environment/data'",
    "awsIotSqlVersion": "2016-03-23",
    "actions": [{ "lambda": { "functionArn": "arn:aws:lambda:us-east-1:077463315120:function:environment_api" } }]
  }
}

# 8. Dashboard

Two interchangeable dashboards are provided. Both poll the same GET /prod/data endpoint every 1 second and render the latest reading plus 30 rows of history.

## 8.1 Zero-build HTML dashboard

Path: dashboard_html/index.html. Single self-contained HTML file with inline CSS and vanilla JavaScript. Double-click to open in any modern browser. No build step, no installs.

## 8.2 React (Vite) dashboard

Path: dashboard/. React 18 + Vite 5. Run npm install then npm run dev for a hot-reload development server, or npm run build to produce a dist/ folder suitable for S3 + CloudFront hosting.

## 8.3 UI features

Status pill (top-right): Live / Stale / Disconnected with colour cues.

Four primary cards: Temperature, Humidity, Air Quality, Fire Detection.

Fire card pulses red on detection (CSS keyframe animation).

Air Quality card colour-shifts on threshold crossings (green/amber/red).

Meta strip: device ID, last reading timestamp, last poll timestamp, errors.

Recent-readings table: last 30 rows newest-first.

Footer: refresh interval and API URL visible to the operator.

# 9. Build & Deployment

## 9.1 Firmware - build and flash

# from the project root
idf.py set-target esp32
idf.py build
idf.py -p COM5 flash monitor       # use your actual COM port

## 9.2 Lambda - upload from PowerShell

cd C:\Users\sai55\OneDrive\Desktop\environment_monitor\lambda
Compress-Archive -Path .\index.mjs -DestinationPath .\environment_api.zip -Force
aws lambda update-function-code `
  --function-name environment_api `
  --zip-file fileb://environment_api.zip `
  --region us-east-1

## 9.3 DynamoDB - one-time create

aws dynamodb create-table `
  --table-name EnvironmentReadings `
  --attribute-definitions AttributeName=device_id,AttributeType=S AttributeName=timestamp,AttributeType=N `
  --key-schema AttributeName=device_id,KeyType=HASH AttributeName=timestamp,KeyType=RANGE `
  --billing-mode PAY_PER_REQUEST `
  --region us-east-1

aws dynamodb update-time-to-live `
  --table-name EnvironmentReadings `
  --time-to-live-specification "Enabled=true, AttributeName=ttl" `
  --region us-east-1

## 9.4 Dashboard - publish to S3 + CloudFront (optional)

cd dashboard
npm install
npm run build
aws s3 mb s3://my-env-monitor-dashboard
aws s3 sync dist/ s3://my-env-monitor-dashboard --delete
aws s3 website s3://my-env-monitor-dashboard --index-document index.html

# 10. Performance & Latency

The system was tuned to deliver sensor-to-dashboard latency under 3 seconds. The dominant constraints are: (a) the DHT11 hardware minimum read interval of 1 s, (b) the TLS handshake overhead per HTTPS POST, and (c) the dashboard's polling interval.

| Stage | Time | Comment |
| --- | --- | --- |
| ESP32 sample (DHT11 + ADC + GPIO) | ~15 ms | Dominated by DHT11's blocking serial read |
| JSON construction (snprintf) | <1 ms | Trivial |
| MQTT publish to broker-ack (QoS 1) | ~1 s | Most of this is TLS keep-alive overhead |
| HTTPS POST including TLS handshake | ~1-3 s | Per-cycle handshake; reused TCP connections would halve this |
| Lambda cold start | ~150 ms (cold), ~50 ms (warm) | Negligible for warm invocations |
| DynamoDB PutItem | ~20 ms | Single-digit-ms p50 typical |
| Dashboard poll interval | 1 s | Configurable in REFRESH_MS |
| End-to-end (sensor read → render) | ~2-3 s | Measured live in operation |

If sub-second end-to-end latency is required, the recommended next step is to drop the HTTPS POST path entirely and rely solely on the MQTT → IoT Rule → Lambda → DynamoDB chain (Section 7.5). MQTT publish + IoT Rule trigger + Lambda + DynamoDB write typically completes inside 500 ms.

# 11. Cost Analysis

All quoted prices are USD list pricing for the us-east-1 region as at project delivery date. Free tier is not subtracted - real bills will be lower for the first 12 months.

| Service | Volume per month per device | Approx. cost (USD) |
| --- | --- | --- |
| AWS IoT Core - messages | 1.3 M MQTT messages (2-sec rate) | $1.30 |
| AWS IoT Core - connections | 30 device-minutes / 100 | negligible |
| API Gateway (HTTP API) | 1.3 M requests | $1.30 |
| AWS Lambda | 1.3 M invocations × ~50 ms × 128 MB | < $0.50 |
| DynamoDB write requests | 1.3 M writes | $1.65 |
| DynamoDB storage | ~1 GB | $0.25 |
| DynamoDB TTL deletes | auto-pruned | free |
| CloudWatch logs | ~5 GB | ~$0.30 |
| Total per device per month |  | ≈ $5.30 |

Scale assumptions: at 100 devices the total is ~$530/month, with no infrastructure operators required.

# 12. Security

Mutual TLS authentication: device-side X.509 cert + AWS-issued client cert.

Least-privilege IoT policy: device can only connect with its client-id pattern and publish to one topic.

Lambda IAM role grants only PutItem/Query/GetItem on the single table.

DynamoDB encryption at rest (default AES-256, AWS-managed key).

TLS in transit for every external hop (MQTT 8883, HTTPS 443).

Wi-Fi credentials and AWS endpoints compiled into firmware, not stored in plaintext on flash.

AmazonRootCA1.pem embedded in firmware - device verifies AWS, not vice versa.

Recommended for production: enable AWS IoT Defender for anomaly detection; rotate device certificates annually.

# 13. Testing & Verification

## 13.1 Smoke tests (operational)

| # | Test | Expected result |
| --- | --- | --- |
| 1 | Power on ESP32 with serial monitor attached | Wi-Fi connects, [MQTT] Connected, sensors initialised |
| 2 | Subscribe to environment/data in AWS IoT MQTT test client | New JSON message every ~2 seconds |
| 3 | Open dashboard in browser | Status pill turns green Live within 3 s; all four cards populated |
| 4 | Breathe on DHT11 sensor | Temperature card increases by 1-2 °C within 2-3 s |
| 5 | Apply flame near flame sensor | Fire Detection card flips to red and pulses within 2-3 s |
| 6 | Wave open marker near MQ135 | Air Quality value rises; card colour shifts amber/red on threshold crossings |
| 7 | Power-cycle Wi-Fi router for 30 s | ESP32 logs retry attempts; reconnects automatically; data resumes flowing |
| 8 | Refresh DynamoDB Explore Items in console | New rows present, newest first, timestamps increasing |
| 9 | curl GET against /prod/data?limit=5 | HTTP 200 with JSON body containing latest 5 readings |
| 10 | Bring multiple browsers to dashboard URL simultaneously | All update independently every 1 s; no race conditions |

## 13.2 Cloud observability

# Tail Lambda logs in real time
aws logs tail /aws/lambda/environment_api --follow --region us-east-1

# Sample DynamoDB rows
aws dynamodb query --table-name EnvironmentReadings `
  --key-condition-expression "device_id = :d" `
  --expression-attribute-values '{":d":{\"S\":\"esp32-env-monitor-01\"}}' `
  --no-scan-index-forward --limit 5 --region us-east-1

# 14. Troubleshooting

| Symptom | Likely cause | Fix |
| --- | --- | --- |
| [MQTT] error: tls_stack=0x... | Certificate not active in AWS IoT, or wrong endpoint | AWS Console → IoT → Security → Certificates → Activate; verify endpoint with aws iot describe-endpoint |
| Wi-Fi connects, MQTT never connects | Network blocks outbound 8883 | Switch to a 4G hotspot to confirm; ask IT to allow outbound 8883 |
| DHT11 read failed: ESP_ERR_TIMEOUT | Missing 10 kΩ pull-up on DATA, or wrong GPIO, or polling faster than 1 s | Add pull-up; verify GPIO 4; SAMPLE_PERIOD_MS ≥ 2000 |
| [API] returned HTTP 404 | API Gateway route not deployed | Click Deploy in API Gateway console; enable Auto-deploy on prod stage |
| [API] returned HTTP 500 | Lambda exception (missing IAM, DynamoDB table missing, syntax error) | aws logs tail to see the real exception, then apply matching fix |
| Dashboard shows Disconnected | CORS not configured, or browser blocks file:// fetch | Configure CORS in API Gateway → Save → Deploy; or serve via python -m http.server 8000 |
| UserCodeSyntaxError in Lambda logs | Smart quotes / BOM introduced during copy-paste into AWS console editor | Use aws lambda update-function-code with a zip file to bypass copy-paste |
| esp-tls: couldn't get hostname | DNS not yet ready, or DHCP failed to push DNS servers | Confirm Got IP appears before MQTT connect; set static DNS as last resort |

# 15. Future Enhancements

| Priority | Feature | Effort | Business value |
| --- | --- | --- | --- |
| High | SMS / email alerts on fire (SNS) | 0.5 day | Real safety value - operators notified anywhere |
| High | Cognito-secured dashboard | 1 day | Restrict who can view plant data |
| Medium | OTA firmware updates | 2 days | Update fleet without on-site visits |
| Medium | Grafana dashboard via IoT Analytics | 1 day | Long-term trend analysis, alerts on rolling averages |
| Medium | Battery + deep-sleep mode | 2 days | Remote locations without mains power |
| Low | LoRaWAN backhaul (no Wi-Fi sites) | 5 days | Reach areas without Wi-Fi coverage |
| Low | Multi-region active-active failover | 3 days | Tolerate full-region AWS outage |
| Low | Modbus / OPC UA bridge | 3 days | Integrate with existing SCADA installs |

# 16. Project Deliverables

The complete project folder is structured as follows:

environment_monitor/
├── CMakeLists.txt                  # ESP-IDF top-level project file
├── sdkconfig.defaults              # Reproducible default config
├── main/                           # ESP-IDF main component
│   ├── CMakeLists.txt
│   ├── idf_component.yml
│   ├── main.c                      # Sensors + MQTT + HTTPS
│   ├── wifi.c, wifi.h              # Wi-Fi station bring-up
│   ├── AmazonRootCA1.pem
│   ├── device.pem.crt
│   └── private.pem.key
├── lambda/
│   ├── index.mjs                   # Production Lambda (Node.js 24)
│   ├── lambda_function.py          # Alternative Python implementation
│   └── README.md
├── aws_setup/
│   ├── iot_policy.json
│   ├── iot_rule_to_lambda.json
│   ├── iot_core_setup.md
│   ├── dynamodb_schema.md
│   └── api_gateway_setup.md
├── dashboard/                      # React (Vite) dashboard
│   ├── index.html, package.json, vite.config.js
│   └── src/App.jsx, App.css, main.jsx, index.css
├── dashboard_html/
│   └── index.html                  # Zero-build dashboard
├── docs/
│   ├── BUILD_AND_FLASH.md
│   ├── DEBUGGING.md
│   ├── DEPLOYMENT.md
│   └── DEMO_GUIDE.md
├── README.md
└── PROJECT_REPORT.docx             # This document

## 16.1 Verification of deliverables

Firmware compiles cleanly with idf.py build (ESP-IDF v6.0.1).

Firmware flashes and runs on hardware, publishing valid JSON to AWS IoT.

Lambda function executes successfully (HTTP 200 responses).

DynamoDB table EnvironmentReadings receives rows in real time.

API Gateway endpoint returns latest readings on GET.

Dashboard renders Live status with all four cards populated.

End-to-end latency measured at ~2-3 seconds in production conditions.

All documentation is current with the deployed system.

End of Document